# ROTAX 912 iS Operators Manual OM

**Document Ref: OM-912iS-ED02**

**Flight Operations, Pre-Flight Run-Up, In-Flight Checklists & Envelope Limitations**

---

# 1. PRE-FLIGHT RUN-UP & INTEGRITY CHECKS

1. **Engine Warm-Up:** Run engine at 2,500 RPM until oil temperature reaches minimum 50 deg C (122 deg F).

2. **Ignition & Lane Cross-Check (4,000 RPM):**

* Select Lane A OFF -> verify Lane B RPM drop < 180 RPM, Delta EGT < 40 deg C.

* Select Lane B OFF -> verify Lane A RPM drop < 180 RPM, Delta EGT < 40 deg C.

3. **Full Power Static Check (Take-Off):** Advance throttle to 100% TPS -> Verify static RPM >= 5,100 RPM, MAP >= 98 kPa, Fuel Flow 24 - 28 L/h, Oil Pressure 3.5 - 5.0 bar.

---

# 2. IN-FLIGHT EMERGENCY CHECKLISTS

* **In-Flight Engine Restart:** Airspeed 80-90 KIAS, Master Switch ON, Lane A & B ON, Fuel Boost Pump ON, Throttle 15% (idle forward).

* **High CHT (> 135 deg C):** Reduce throttle to 4,600 RPM, command fuel trim enrichment +12%, verify cowl flap open, descend 3,000 ft into denser ambient air.

* **Low Oil Pressure (< 2.0 bar):** Reduce throttle to 4,200 RPM, monitor oil temperature trend, declare urgency, vector toward recovery waypoint.

* **Electrical Bus Voltage Sag (< 12.8 V):** Shed non-essential avionics and ISR radar payloads, verify battery discharge current < 5 A.